# RADAR SYSTEM USING ARDUINO, SERVO MOTOR, AND ULTRASONIC SENSOR

# 1. Introduction

The objective of this project is to design a simple radar system that can detect obstacles using an ultrasonic sensor mounted on a servo motor. The system is controlled using an Arduino Uno R3, and real-time visual representation is displayed using Processing IDE. This project demonstrates basic principles of sonar-based object detection and graphical visualization.

# 2. Components Used

| Component | Quantity | Description |
| --- | --- | --- |
| Arduino Uno R3 | 1 | Microcontroller board for control logic |
| Ultrasonic Sensor HC-SR04 | 1 | Used for distance measurement |
| Servo Motor SG90 | 1 | Rotates the sensor to scan area |
| Breadboard & Wires | As needed | For circuit connections |
| USB Cable | 1 | To upload code and provide power |
| Computer (Processing IDE) | 1 | To visualize data from Arduino |

# 3. Working Principle

The servo motor rotates from 0° to 180° and back, carrying the ultrasonic sensor. At each angle, the ultrasonic sensor sends out a pulse and listens for the echo. The time taken for the echo to return is used to calculate the distance of an object.

These distance values along with the corresponding angle are sent to the computer via serial communication, where the Processing program receives the data and plots it on a radar-style display in real time.

# 4. Circuit Diagram

Connections:

HC-SR04:

VCC → 5V on Arduino

GND → GND on Arduino

TRIG → Pin 9

ECHO → Pin 10

SG90 Servo:

VCC (Red) → 5V on Arduino

GND (Brown) → GND on Arduino

Signal (Orange) → Pin 11

# 5. Arduino Code Summary

• The servo motor sweeps from 0° to 180° and back.
• At each angle, the HC-SR04 sensor sends a signal and measures distance.
• The angle and distance are sent via the serial port in the format angle,distance.

# 6. Processing Code Summary

• The Processing sketch reads serial data from Arduino.
• The angle and distance are parsed and used to draw:
   - Rotating radar lines
   - Detected objects as red dots/lines
   - Angle and distance information on-screen
• The system runs in full screen for immersive visualization.

# 7. Applications

• Object detection and mapping
• Miniature radar simulation
• Educational demonstrations of sonar principles
• Foundation for autonomous robot navigation systems

# 8. Conclusion

This project successfully demonstrates a basic radar system using accessible electronic components. It provides hands-on experience with Arduino programming, servo control, ultrasonic sensing, and real-time data visualization using Processing.

# 9. Future Enhancements

• Add multiple sensors for 360° scanning
• Save and replay scan data
• Add buzzer/alarm for close-range alerts
• Incorporate wireless data transmission (e.g., Bluetooth or Wi-Fi)